JB

things you might need to know about le big block you might have

forgotten:

Alum heads, flow 330cfm intake and 280cfm exaust

10-1 cr (kinda wanna keep that but don't "have to")

would like to run a blow thru or carb in a box (like vortec

style systems)

carb size is 850cfm, but usualy flows a little more (like 10%

more)

hp peak na is at 5900-6000, kinda wanna stay there too

goal: 900+...1000 would make my nuts tingle ...but 900 would

make me smile too (at the flywheel)

Let me know brotha!

bore: 4.280

crank: 4340 stroke: 4.25

Rods 4340 h beams: 6.385

4 bolts main caps, studed

head studs

deck to head clerance: 0

head gasket thickness: 0.041

combustion chamber cc: 118

piston: SRP/JE forged dome: 18cc

cr: right at 10-1

Rings: speed pro moly, hand gaped for nitrous use

intake valve: 2.25

Exaust valve: 1.88

rocker ratio: 1.7 (int/ex)

Cam: Comp Cam Extreme Energy 292 solid roller 254/260 @50, 640 lift 110

lobe(exact specs are online).

valves are Manley, with a Menolite coating

carb: 850cfm speed demon

fuel pump: engine driven mech. 130gph (would upgrade to elec.)

ingnition: MSD 6AL/MSD Pro Billet dist. MSD blaster coil

Trany: th400, 2500 stall (will upgrade to about 4000 stall)

gears: 3.73's (will upgrade to about 4.11 or 4.56)

tires: 26" tall (to be changed to 28")

I was thinking about using the Vortec Carb box too.

and use shorty headers upside down to keep things easier

twin turbos sounds naudy to me, and I like naudy stuff!

CID: (4.28/2)^2 * 3.14 * 4.25 = 489.16

CR= 1 +   0.7845 x bore^2 x stroke

CCV+HGV+PDV

CCV=comb chamber volume

HGV= HGCT x 0.7845 x BORE^2

PDV= (0.7845 x BORE^2 x DPD) + (VPD – VPB)

HGCT= head gasket compressed thickness

VE  =  volumetric efficiency

CFM  =  engine air flow rate - cubic feet per minute

CID  =  engine displacement, size or volume - cubic inches

RPM  =  engine speed - revolutions per minute

NOC  =  number of cylinders

BORE  =  length

STROKE  =  length

HGV  =  head gasket volume

HGCT  =  head gasket compressed thickness

PDV  =  piston deck volume

DPD  =  deck to piston distance

VPD  =  volume of piston depressions

VPB  =  volume of piston bumps

SV  =  swept volume

CR  =  compression ratio

CCV  =  combustion chamber volume

ISH  =  injector size per horsepower

HP  =  horsepower

CFM = CID x RPM x VE

3456