Stoichiometric Ratio = 14.7 corresponds to 3.3V

MONITOR DESCRIPTION

Under the air–fuel ratio feedback control, if the A/F sensor voltage output indicates rich or lean for a certain

period of time, the ECM determines that there is a malfunction in the A/F sensor. The ECM illuminates the

MIL and sets a DTC.

Example:

If the A/F sensor voltage output is less than 2.8 V (very rich condition) for 10 seconds, despite the HO2 sensor

voltage output being less than 0.85 V, the ECM sets DTC P2196. Alternatively, if the A/F sensor voltage

output is more than 3.8 V (very lean condition) for 10 seconds, despite the HO2 sensor voltage output being

0.15 V or more, DTC P2195 is set.

P2195 (Lean side malfunction):

Time while all of following conditions met 	2 seconds or more

Rear HO2 sensor voltage 			0.15 V or more

Time after engine start 			30 seconds or more

A/F sensor status 				Activated

Fuel system status				 Closed–loop

Engine 					Running

Sub–feedback status			 Executing

P2196 (Rich side malfunction):

Time while all of following conditions met	 2 seconds or more

Rear HO2 sensor voltage			 Below 0.85 V

Time after engine start 			30 seconds or more

A/F sensor status 				Activated

Fuel system status 				Closed–loop

Engine 					Running

Sub–feedback status 			Executing

When the vehicle is driven:

- The output voltage of the A/F sensor may be below 2.8 V (0.56 V*) during fuel enrichment. For the vehicle, this translates to a sudden increase in speed with the accelerator pedal fully depressed when trying to overtake another vehicle. The A/F sensor is functioning normally.

- The A/F sensor is a current output element; therefore, the current is converted into a voltage inside the ECM. Measuring the voltage at the connectors of the A/F sensor or ECM will show a constant voltage result.