Typical Voltage: 0.475V + 0.025V

TYPICAL ENABLING CONDITIONS

Accumulated time that all following conditions met 			30 seconds

Battery voltage 						11 V or more

Intake air temperature					 –10_C (14_F) or more

Idle							 OFF

Mass air flow rate						35 g/sec

Engine RPM						than 4,500 rpm

Engine coolant temperature sensor 				 (167_F) or more

Fuel system status 						Closed Loop

A/F sensor 						Activated

HO2 sensor						 Activated

Estimated catalyst temperature 				Both of following conditions 1 and 2 met

1. Up stream catalyst temperature				 550 to 900_C (1,022 to 1,652_F)

2. Down stream catalyst temperature 				550 to 900_C (1,022 to 1,652_F)

CTech Notes:

- A/F sensor used for fuel trims

- Heated O2 sensor used for checking condition of catalytic converter only.

- Standard electrical potential output of heated oxygen sensor corresponding with properly working catalytic converter is ~3.3V.

- Wideband Oxygen sensor appears to not be usable for tuning due to the calibration range being very lean of target lambda values for forced induction tuning.

- Heated Oxygen sensor is a typical voltage-range type.