Concerns

AFR vs O2 sensor

TPS vs Pedal Position Sensor

MAF Signal

Knock sensor: frequency and logic

Leak Detection Pump Assembly

Datalink

Ignition Signal

Injector Signal

A/F vs O2 sensor

Primary Concern: What is the difference between the upstream[ [pre-catalytic converter] and downstream [post-catalytic coverter] and their signals?

Secondary: Is the signal from the 5-wire wideband A/F sensor usable for tuning?

Secondary: How does the computer analyze the signal from the A/F sensor and how does it process the fuel trims based on this information.

A/F sensor:

Stoichiometric Ratio = 14.7 corresponds to 3.3V

MONITOR DESCRIPTION

Under the air–fuel ratio feedback control, if the A/F sensor voltage output indicates rich or lean for a certain

period of time, the ECM determines that there is a malfunction in the A/F sensor. The ECM illuminates the

MIL and sets a DTC.

Example:

If the A/F sensor voltage output is less than 2.8 V (very rich condition) for 10 seconds, despite the HO2 sensor

voltage output being less than 0.85 V, the ECM sets DTC P2196. Alternatively, if the A/F sensor voltage

output is more than 3.8 V (very lean condition) for 10 seconds, despite the HO2 sensor voltage output being

0.15 V or more, DTC P2195 is set.

P2195 (Lean side malfunction):

Time while all of following conditions met 	2 seconds or more

Rear HO2 sensor voltage 			0.15 V or more

Time after engine start 			30 seconds or more

A/F sensor status 				Activated

Fuel system status				 Closed–loop

Engine 					Running

Sub–feedback status			 Executing

P2196 (Rich side malfunction):

Time while all of following conditions met	 2 seconds or more

Rear HO2 sensor voltage			 Below 0.85 V

Time after engine start 			30 seconds or more

A/F sensor status 				Activated

Fuel system status 				Closed–loop

Engine 					Running

Sub–feedback status 			Executing

When the vehicle is driven:

- The output voltage of the A/F sensor may be below 2.8 V (0.56 V*) during fuel enrichment. For the vehicle, this translates to a sudden increase in speed with the accelerator pedal fully depressed when trying to overtake another vehicle. The A/F sensor is functioning normally.

- The A/F sensor is a current output element; therefore, the current is converted into a voltage inside the ECM. Measuring the voltage at the connectors of the A/F sensor or ECM will show a constant voltage result.

Heated O2 sensor:

Typical Voltage: 0.475V + 0.025V

TYPICAL ENABLING CONDITIONS

Accumulated time that all following conditions met 			30 seconds

Battery voltage 						11 V or more

Intake air temperature					 –10_C (14_F) or more

Idle							 OFF

Mass air flow rate						35 g/sec

Engine RPM						than 4,500 rpm

Engine coolant temperature sensor 				 (167_F) or more

Fuel system status 						Closed Loop

A/F sensor 						Activated

HO2 sensor						 Activated

Estimated catalyst temperature 				Both of following conditions 1 and 2 met

1. Up stream catalyst temperature				 550 to 900_C (1,022 to 1,652_F)

2. Down stream catalyst temperature 				550 to 900_C (1,022 to 1,652_F)

CTech Notes:

- A/F sensor used for fuel trims

- Heated O2 sensor used for checking condition of catalytic converter only.

- Standard electrical potential output of heated oxygen sensor corresponding with properly working catalytic converter is ~3.3V.

- Wideband Oxygen sensor appears to not be usable for tuning due to the calibration range being very lean of target lambda values for forced induction tuning.

- Heated Oxygen sensor is a typical voltage-range type.

- AFR sensor signal is current-bases, constant voltage, hence not usable before ECM as a tuning signal.

TPS vs Pedal Position

Primary concern: Which is use for fuel trims and load values.

Primary Concern: Which has the best granularity.

Secondary Concern: Which is best applicable to a standard TPS input on an electronic

tuning device

Solution: VTA2. Actual throttle POS sensor, primary sensor to give best granularity, 0.5-4.5V

Ctech Notes:

- VTA1 has the best granularity, 0.5..4.5V

- Measured voltage range of VPA1 and VPA2 were between 2.5 and 3.0V.

MAF signal

Knock Sensor

Leak Detection

PPMP Pump module

Cam timing control valve

Datalink

WFSE

TC

SIL

TAL

SG

CG