Installation Notes:

Differential Plug

The Viton O-ring has a clearance fit of 0.006” + 0.003”. Apply some heavy lubricant (motor oil or gear oil) to the seal and to the journal walls on the differential. Place the spacer and hold-down fixture, lock-washer and bolt on and torque the bolt to 20-ft/lb

Steering Rack Fluid Plugs

The front steering rack has 2 hard lines running along the subframe. They connect to 2 soft rubber lines with inverted flare fittings. Separate the fitting from the rubber line to the hard line; not from the hard line to the steering rack. Use a 17mm and a 14mm line wrench. Do not use a standard wrench or crescent wrench, it will strip the fittings. If the fittings are stubborn, apply some penetrating lubricant (PB Blaster works well) and wait some time for it to travel through the fitting; then reattempt to separate the fitting. The application of heat using a propane torch or heat gun may also help separate the fittings. When the fittings have been separated, clean the male plug and make sure no debris is on the face of the flare. Put the AN cap on the metric adapter and torque that fitting to 12-14 ft-lb. Use no lubricant or sealing tape, the flare makes the seal. Ensure the flare of the vehicle’s steering rack lines are not damaged. Thread the metric fitting onto the male threaded fitting on the hard-pipe sourced from the steering rack. Check that the threads are not crossed and tighten the fittings by hand until the flare bottoms out on the adapter. Torque this connection to 38 ft-lb. Repeat for the other line. Supervise this connection by starting the engine and turning the steering wheel to apply pressure to the steering rack. If leaks occur, tighten the fittings until the seal is complete. Do not use thread locking agents or pipe dope on this connection unless absolutely necessary.

Tie Rods

The washers, bolts, stop nuts and radius rods all have a specific installation method. The blue Metric nuts have a crowned side and a flat side; the flat side should be directed towards the tension face. The washers also have a crown and flat side, and the flat side should similarly be directed towards the tension face. The radius rods have a specific inboard/outboard direction according to the left and right threading of the spherical bearing ends. The knurled portion of the radius rod should be closer to the outboard side of the suspension, and the spherical bearing end associated with this knurled section should be fastened to the trailing arm on the same attachment lug as the All-Wheel-Steering rack.

To install the blue metric bolt and nut onto the trailing arm, assemble the components like so:

Top

bolt head

flat washer, crowned up

rod end

flat washer, crowned up

Trailing arm lug

flat washer, crowned down

locking washer

blue nut, crowned down

Use a deep socketed 19mm ratchet and torque wrench to tighten the bolt. Secure nut and torque bolt and use no lubricant or locking compound.

To install the zinc-steel bolt into the sub-frame, assemble the components like so:

bolt head

locking washer

flat washer, crowned up

rod end

flat washer, crowned up

subframe

Use a torque wrench on the bolt head and use no lubricant or locking compound. Ensure threads are clean and undamaged before installing hardware.

Dry* Torque Specifications :

Zinc 8.8 Metric Bolts attached to sub-frame:

Din 961/ISO8676 40mm shank length = 54.5 lb*ft

Blue Metric Bolts attached to Trailing arm:

ISO A490 Class 10.9 50mm shank length = 63.50 lb*ft

* For lubricated torque settings contact CTech.

Crowned Nut, Crowned Side up:

Crowned Washer, Crowned Side up depicted on right:

Installation of Fluid caps:

Installation of radius rods:

Image1: Zinc-Steel bolt and right-hand-threaded rod end:

Image 2: Blue Metric Bolt and Rod end:

Image 3 & 4: Installed Assembly: